*---------------------------------------------------------------------*

*---------------------------------------------------------------------*

*This program solves the 2D Advection Equation

*                  dc/dt + d/dx(cu) + d/dy(cv) = 0

*on a square domain using Periodic B.C.'s

*in both x and y and using Semi-Implicit Semi-Lagrangian

*Bilinear Rectangular Finite Elements.

*Written by F.X. Giraldo on 3/95

*           NRC Fellow

*           Department of Mathematics

*           Naval Postgraduate School

*           Monterey, CA 93940

*---------------------------------------------------------------------*

*---------------------------------------------------------------------*

program slt_advect

implicit real*8(a-h,o-z)

parameter ( imax=21 )

c

c mxpoi  max number of points

c mxele  max number of elements

c mxbou  max number of boundary points

c nd     max number of vertices for each elements

c

parameter ( mx=imax*imax, mxpoi=mx, mxele=mx, mxbou=mx/5, nd=4 )

c

c     global matrices

c

dimension a(mxpoi,mxpoi), b(mxpoi)

dimension f(mxpoi)

dimension coord(mxpoi,2)

dimension cmat(mxpoi)

integer intma(mxele,nd), iboun(mxbou,4)

c

c     u velocity arrays

c

dimension um(mxpoi),      u0(mxpoi),     up(mxpoi)

dimension                 u0_x(mxpoi)

c

c     v velocity arrays

c

dimension vm(mxpoi),      v0(mxpoi),     vp(mxpoi)

dimension                 v0_y(mxpoi)

c

c     phi arrays

c

dimension phim(mxpoi),    phi0(mxpoi),   phip(mxpoi)

c

c     departure point arrays

c

dimension alfm(mxpoi,2),  alf0(mxpoi,2)

c

c     spline derivative arrays

c

dimension dphim(mxpoi)

dimension du0(mxpoi), du0_x(mxpoi)

dimension dv0(mxpoi), dv0_y(mxpoi)

c

c     Auxiliary Matrices

c

integer node(imax,imax)

c

c     Read the Input Variables and create the Grid

c

c   input file contains 4 lines

c on first: number of grid points in x (nx) and y (ny) direction

c on second:range of x values (xmin, xmax),

c           range of y values (ymin,ymax)

c on third: delta t,

c            final time of integration (time_final),

c            number of times steps between plots (iplot)

c

call init(phi0,u0,v0,node,coord,alf0,intma,iboun,npoin,

$          nelem,nboun,xmin,xmax,ymin,ymax,nx,ny,dx,dy,dt,

$          ntime,ym,nd,mxpoi,mxele,mxbou,imax,iplot)

c

c     Construct the Lumped Mass Matrix used for the derivative computation

c

call get_geom(nelem,npoin,intma,coord,cmat,node,

$              nx,ny,nd,dx,dy,mxpoi,mxele,imax)

time=0.0

pi=4.0*atan(1.0)

open(1,file='matlab.out')

c

c isets = number of time steps at which solution is plotted

c

isets=ntime/iplot + 2

write(1,*)isets

c

c  always plot initial condition

c

call output(phi0,u0,v0,npoin,time,nx,ny,mxpoi)

c

c     begin the time marching

c

c     Do the 1st Time-step Integration to get 2-time levels

c

do itime=1,1

time=time + dt

dtime=time/(2.0*pi)

write(*,'(" timestep time = ",i5,2x,e12.4)')itime,dtime

call matsuno(phim,phi0,phip,um,u0,up,

$                vm,v0,vp,coord,intma,npoin,nelem,dt,dx,dy,

$                node,nx,ny,ym,nd,mxpoi,mxele,imax)

call update(phim,phi0,phip,um,u0,up,vm,v0,vp,alfm,alf0,

$               npoin,mxpoi)

end do

c

c     TIME MARCH

c

do itime=2,ntime

time=time + dt

dtime=time/(2.0*pi)

write(*,'(" timestep time = ",i5,2x,e12.4)')itime,dtime

*

*        1st, DETERMINE DEPARTURE POINT

*

call splie2(du0,u0,coord,nx,ny,node,mxpoi,imax)

call splie2(dv0,v0,coord,nx,ny,node,mxpoi,imax)

call depart(alf0,alfm,coord,u0,du0,v0,dv0,

$               npoin,dt,xmin,xmax,ymin,ymax,

$               nd,mxpoi,node,nx,ny,imax)

*

*        2nd, COMPUTE DERIVATIVES

*

call deriv_x(u0_x,u0,intma,cmat,node,npoin,nelem,

$                nx,ny,nd,dy,mxpoi,mxele,imax)

call deriv_y(v0_y,v0,intma,cmat,node,npoin,nelem,

$                nx,ny,nd,dx,mxpoi,mxele,imax)

*

*        3rd, INTERPOLATE PHIM, UM_X=U0_X, VM_Y=V0_Y

*        at the departure point = X - 2*ALPHA

*

call splie2(dphim,phim,coord,nx,ny,node,mxpoi,imax)

call splie2(du0_x,u0_x,coord,nx,ny,node,mxpoi,imax)

call splie2(dv0_y,v0_y,coord,nx,ny,node,mxpoi,imax)

c

c        INTERPOLATE THE RIGHT HAND SIDE FUNCTION

c

call interp(f,phim,u0_x,v0_y,dphim,du0_x,dv0_y,coord,

$               alf0,node,npoin,dt,xmin,xmax,ymin,ymax,ym,

$               nx,ny,nd,mxpoi,imax)

do ip=1,npoin

phip(ip)=f(ip)

up(ip)=u0(ip)

vp(ip)=v0(ip)

end do

c

c        Update the values

c

call update(phim,phi0,phip,um,u0,up,vm,v0,vp,alfm,alf0,

$               npoin,mxpoi)

c

c     check time for printing output

c

if (mod(itime,iplot).eq.0)

$      call output(phi0,u0,v0,npoin,time,nx,ny,mxpoi)

end do

1000 continue

call output(phi0,u0,v0,npoin,time,nx,ny,mxpoi)

close(1)

stop

end

*---------------------------------------------------------------------*

*This subroutine finds the departure point

* ALPHA1=DT*U(X-ALPHA1,Y-ALPHA2,T) ALPHA2=DT*V(X-ALPHA1,Y-ALPHA2,T)

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine depart(alf0,alfm,coord,u0,du0,v0,dv0,

$                  npoin,dt,xmin,xmax,ymin,ymax,

$                  nd,mxpoi,node,nx,ny,imax)

implicit real*8(a-h,o-z)

dimension alf0(mxpoi,2), alfm(mxpoi,2)

dimension u0(mxpoi), du0(mxpoi)

dimension v0(mxpoi), dv0(mxpoi)

dimension coord(mxpoi,2)

integer node(imax,imax)

do ip=1,npoin

alpha1=alfm(ip,1)

alpha2=alfm(ip,2)

do k=1,3

xd=coord(ip,1) - alpha1

yd=coord(ip,2) - alpha2

if (xd.lt.xmin) xd=xmax - (xmin - xd)

if (xd.gt.xmax) xd=xmin + (xd - xmax)

if (yd.lt.ymin) yd=ymax - (ymin - yd)

if (yd.gt.ymax) yd=ymin + (yd - ymax)

if ( (xd.lt.xmin.or.xd.gt.xmax) .or.

$           (yd.lt.ymin.or.yd.gt.ymax) ) then

print*,' Error in DEPART'

print*,'XD out of Range = ',xd,yd

print*,'ip alpha = ',ip,alpha1,alpha2

stop

endif

call splin2(u,coord,u0,du0,xd,yd,nx,ny,node,mxpoi,imax)

call splin2(v,coord,v0,dv0,xd,yd,nx,ny,node,mxpoi,imax)

alpha1=dt*u

alpha2=dt*v

end do

alf0(ip,1)=alpha1

alf0(ip,2)=alpha2

end do

return

end

*---------------------------------------------------------------------*

*This subroutine computes the 4th Order Accurate derivative WRT X of the

*variable UNKNO and stores it in DERIP using Bilinear Rectangular

*Finite Elements

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine deriv_x(derip,unkno,intma,cmat,node,npoin,nelem,

$                   nx,ny,nd,dy,mxpoi,mxele,imax)

implicit real*8(a-h,o-z)

c

c     global arrays

c

dimension derip(mxpoi), unkno(mxpoi)

dimension cmat(mxpoi)

integer intma(mxele,nd), node(imax,imax)

c

c     local coordinate system for a CCW ordered rectangle

c

dimension xi(4), eta(4)

data xi  / -1, 1, 1,-1 /

data eta / -1,-1, 1, 1 /

c

c     initialize arrays

c

do ip=1,npoin

derip(ip)=0.0

end do

c

c     Loop thru the Elements and find the 1St DERIVATIVES

c

do ie=1,nelem

do i=1,nd

phi_x=0.0

ip=intma(ie,i)

do k=1,nd

phik=unkno(intma(ie,k))

phi_x=phi_x + xi(k)*phik*( 2.0 + 2.0/3.0*eta(k)*eta(i) )

end do

derip(ip)=derip(ip) + dy/16.0*phi_x

end do

end do

c

c     Account for Periodicity

c

do j=1,ny

i1=node(1,j)

inx=node(nx,j)

derip_temp=derip(i1) + derip(inx)

derip(i1)=derip_temp

derip(inx)=derip_temp

end do

c

c     Now multiply by the Inverse of the Lumped mass matrix

c

do ip=1,npoin

derip(ip)=derip(ip)*cmat(ip)

end do

return

end

*---------------------------------------------------------------------*

*This subroutine computes the 4th Order Accurate derivative WRT Y of the

*variable UNKNO and stores it in DERIP using Bilinear Rectangular

*Finite Elements

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine deriv_y(derip,unkno,intma,cmat,node,npoin,nelem,

$                   nx,ny,nd,dx,mxpoi,mxele,imax)

implicit real*8(a-h,o-z)

c

c     global arrays

c

dimension derip(mxpoi), unkno(mxpoi)

dimension cmat(mxpoi)

integer intma(mxele,nd), node(imax,imax)

c

c     local coordinate system for a CCW ordered rectangle

c

dimension xi(4), eta(4)

data xi  / -1, 1, 1,-1 /

data eta / -1,-1, 1, 1 /

c

c     initialize arrays

c

do ip=1,npoin

derip(ip)=0.0

end do

c

c     Loop thru the Elements and find the 1St DERIVATIVES

c

do ie=1,nelem

do i=1,nd

phi_y=0.0

ip=intma(ie,i)

do k=1,nd

phik=unkno(intma(ie,k))

phi_y=phi_y + eta(k)*phik*( 2.0 + 2.0/3.0*xi(k)*xi(i) )

end do

derip(ip)=derip(ip) + dx/16.0*phi_y

end do

end do

c

c     Account for Periodicity

c

do j=1,ny

i1=node(1,j)

inx=node(nx,j)

derip_temp=derip(i1) + derip(inx)

derip(i1)=derip_temp

derip(inx)=derip_temp

end do

c

c     Now multiply by the Inverse of the Lumped mass matrix

c

do ip=1,npoin

derip(ip)=derip(ip)*cmat(ip)

end do

return

end

*---------------------------------------------------------------------*

*This subroutine computes the Inverse Lumped Mass Matrix

*for Bilinear Rectangular Finite Elements used for obtaining the

*4th Order Accurate 1st and 2nd derivatives in both X and Y.

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine get_geom(nelem,npoin,intma,coord,cmat,node,

$                    nx,ny,nd,dx,dy,mxpoi,mxele,imax)

implicit real*8(a-h,o-z)

c

c     global arrays

c

dimension coord(mxpoi,2), cmat(mxpoi)

integer intma(mxele,nd), node(imax,imax)

c

c     Compute the inverse lumped mass matrix

c

do ip=1,npoin

cmat(ip)=0.0

end do

do ie=1,nelem

do in=1,nd

ip=intma(ie,in)

cmat(ip)=cmat(ip) + dx*dy/4.0

end do

end do

c

c     Account for Periodicity in X

c

do j=1,ny

i1=node(1,j)

inx=node(nx,j)

cmat_temp=cmat(i1) + cmat(inx)

cmat(i1)=cmat_temp

cmat(inx)=cmat_temp

end do

c

c     Invert the lumped mass matrix

c

do ip=1,npoin

cmat(ip)=1.0/cmat(ip)

end do

return

end

*---------------------------------------------------------------------*

*This subroutine reads in the input file.

*The info read is the number of grid points (in x and y), the domain,

*the time step, the final time, and the number of time steps for plotting.

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine init(phi0,u0,v0,node,coord,alf0,intma,iboun,npoin,

$                nelem,nboun,xmin,xmax,ymin,ymax,nx,ny,dx,dy,dt,

$                ntime,ym,nd,mxpoi,mxele,mxbou,imax,iplot)

implicit real*8(a-h,o-z)

dimension coord(mxpoi,2), alf0(mxpoi,2)

dimension phi0(mxpoi), u0(mxpoi), v0(mxpoi)

integer intma(mxele,nd), iboun(mxbou,4), node(imax,imax)

c

c     Read Input File

c

read(*,*)nx,ny

read(*,*)xmin,xmax,ymin,ymax

read(*,*)dt,time_final,iplot

c

c     check bounds

c

if (nx*ny.gt.mxpoi) then

write(*,'(" Error! - Need to Enlarge MXPOI")')

stop

else if ((nx-1)*(ny-1).gt.mxele) then

write(*,'(" Error! - Need to Enlarge MXELE")')

stop

else if (2*(nx-1)+2*(ny-1).gt.mxbou) then

write(*,'(" Error! - Need to Enlarge MXBOU")')

stop

endif

c

c     set some constants

c

pi=4.0*atan(1.0)

ntime=nint(time_final/dt)

dt=dt*2.0*pi

time_final=time_final*2.0*pi

xm=0.5*(xmax+xmin)

ym=0.5*(ymax+ymin)

dx=(xmax-xmin)/(nx-1)

dy=(ymax-ymin)/(ny-1)

xl=xmax-xmin

yl=ymax-ymin

w=1.0

cx=0.25*xl

cy=0.50*yl

h=100.0

rc=0.125*xl

velmax=-1e5

c

c     set the Initial Conditions

c

ip=0

do j=1,ny

y=ymin + real(j-1)*dy

do i=1,nx

x=xmin + real(i-1)*dx

ip=ip+1

r=sqrt( (x-cx)**2 + (y-cy)**2 )

phi0(ip)=0.0

if (r.lt.rc) then

phi0(ip)=h*(1.0-r/rc)

endif

u0(ip)=+(y-ym)

v0(ip)=-(x-xm)

vel1=u0(ip)**2 + v0(ip)**2

velmax=max(velmax,vel1)

end do

end do

cfl=dt*sqrt(velmax/(dx**2 + dy**2))

print*,' ** CFL = ',cfl

c

c     GENERATE COORD

c

npoin=nx*ny

ip=0

do j=1,ny

do i=1,nx

ip=ip+1

node(i,j)=ip

coord(ip,1)=xi + dx*real(i-1)

coord(ip,2)=yi + dy*real(j-1)

alf0(ip,1)=0.5*dx

alf0(ip,2)=0.5*dy

end do

end do

c

c     GENERATE INTMA

c

nelem=(nx-1)*(ny-1)

ie=0

do j=1,ny-1

do i=1,nx-1

ie=ie+1

intma(ie,1)=node(i,j)

intma(ie,2)=node(i+1,j)

intma(ie,3)=node(i+1,j+1)

intma(ie,4)=node(i,j+1)

end do

end do

c

c     GENERATE IBOUN

c

nboun=2*(nx-1)

ib=0

c     bottom (y=ymin)

ie=1

do i=1,nx-1

ib=ib+1

iboun(ib,1)=node(i,1)

iboun(ib,2)=node(i+1,1)

iboun(ib,3)=ie

iboun(ib,4)=3

ie=ie+1

end do

c     top (y=ymax)

ielem=nelem

do i=nx,2,-1

ib=ib+1

iboun(ib,1)=node(i,ny)

iboun(ib,2)=node(i-1,ny)

iboun(ib,3)=ielem

iboun(ib,4)=3

ielem=ielem-1

end do

return

end

*---------------------------------------------------------------------*

*This subroutine interpolates the right hand side function for the

*2D Advection Equation using a 3-time level Semi-Lagrangian

*Semi-Implicit Method

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine interp(f,phim,u0_x,v0_y,dphim,du0_x,dv0_y,coord,

$               alf0,node,npoin,dt,xmin,xmax,ymin,ymax,ym,

$               nx,ny,nd,mxpoi,imax)

implicit real*8(a-h,o-z)

dimension f(mxpoi)

dimension phim(mxpoi), u0_x(mxpoi), v0_y(mxpoi)

dimension dphim(mxpoi),du0_x(mxpoi),dv0_y(mxpoi)

dimension coord(mxpoi,2), alf0(mxpoi,2)

integer node(imax,imax)

do ip=1,npoin

c

c     1st, Interpolate "-" values = F( x - 2*alpha, t - dt )

c

xd=coord(ip,1) - 2.0*alf0(ip,1)

yd=coord(ip,2) - 2.0*alf0(ip,2)

if (xd.lt.xmin) xd=xmax - (xmin - xd)

if (xd.gt.xmax) xd=xmin + (xd - xmax)

if (yd.lt.ymin) yd=ymax - (ymin - yd)

if (yd.gt.ymax) yd=ymin + (yd - ymax)

if ( (xd.lt.xmin.or.xd.gt.xmax) .or.

$        (yd.lt.ymin.or.yd.gt.ymax) ) then

print*,' Error in INTERP'

print*,'XD out of Range = ',xd,yd

stop

endif

c        Do Interpolation

call splin2(phi,coord,phim,dphim,xd,yd,nx,ny,node,mxpoi,imax)

call splin2(u_x,coord,u0_x,du0_x,xd,yd,nx,ny,node,mxpoi,imax)

call splin2(v_y,coord,v0_y,dv0_y,xd,yd,nx,ny,node,mxpoi,imax)

f(ip)=(1.0-dt*(u_x + v_y))/(1.0+dt*(u0_x(ip) + v0_y(ip)))*phi

end do

return

end

*---------------------------------------------------------------------*

*This subroutine solves the 2D Advection Equation

*using the Backward Euler with a predictor-corrector strategy

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine matsuno(phim,phi0,phip,um,u0,up,

$                   vm,v0,vp,coord,intma,npoin,nelem,dt,dx,dy,

$                   node,nx,ny,ym,nd,mxpoi,mxele,imax)

implicit real*8(a-h,o-z)

dimension phim(mxpoi), phi0(mxpoi), phip(mxpoi)

dimension um(mxpoi), u0(mxpoi), up(mxpoi)

dimension vm(mxpoi), v0(mxpoi), vp(mxpoi)

dimension coord(mxpoi,2)

integer intma(mxele,nd), node(imax,imax)

c

c     Loop through the points and integrate using Forward Time

c     and Centered Space...

c

*     Predictor Stage (forward Euler)

do j=1,ny

j1=j-1

j2=j+1

if (j1.lt.1) j1=ny-1

if (j2.gt.ny) j2=2

do i=1,nx

i1=i-1

i2=i+1

if (i1.lt.1) i1=nx-1

if (i2.gt.nx) i2=2

c

c           Set up the nodes in X and Y

c

ip=node(i,j)

ip1=node(i1,j)

ip2=node(i2,j)

jp1=node(i,j1)

jp2=node(i,j2)

c

c           integrate PHI

c

phim(ip)=phi0(ip)

$    -0.5*dt/dx*u0(ip)*( phi0(ip2)-phi0(ip1) )

$    -0.5*dt/dy*v0(ip)*( phi0(jp2)-phi0(jp1) )

c

c        integrate U

c

um(ip)=u0(ip)

c

c        integrate V

c

vm(ip)=v0(ip)

end do

end do

*     Corrector Stage (backward Euler)

do j=1,ny

j1=j-1

j2=j+1

if (j1.lt.1) j1=ny-1

if (j2.gt.ny) j2=2

do i=1,nx

i1=i-1

i2=i+1

if (i1.lt.1) i1=nx-1

if (i2.gt.nx) i2=2

c

c           Set up the nodes in X and Y

c

ip=node(i,j)

ip1=node(i1,j)

ip2=node(i2,j)

jp1=node(i,j1)

jp2=node(i,j2)

c

c           integrate PHI

c

phip(ip)=phi0(ip)

$    -0.5*dt/dx*um(ip)*( phim(ip2)-phim(ip1) )

$    -0.5*dt/dy*vm(ip)*( phim(jp2)-phim(jp1) )

c

c        integrate U

c

up(ip)=u0(ip)

c

c        integrate V

c

vp(ip)=v0(ip)

end do

end do

c

c     Apply the Periodic Boundary Conditions

c

do j=1,ny

i1=node(1,j)

i2=node(nx,j)

phip(i1)=phip(i2)

up(i1)=up(i2)

vp(i1)=vp(i2)

end do

do i=1,nx

i1=node(i,1)

i2=node(i,ny)

phip(i1)=phip(i2)

up(i1)=up(i2)

vp(i1)=vp(i2)

end do

1000  continue

return

end

*---------------------------------------------------------------------*

*This subroutine writes the output.  It is currently set only to

*print the concentration (or color) function at each node point.

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine output(phi,u,v,npoin,time,nx,ny,mxpoi)

implicit real*8(a-h,o-z)

dimension phi(mxpoi), u(mxpoi), v(mxpoi)

pi=4.0*atan(1.0)

write(1,'(2(i6,1x),e16.8)'),nx,ny,time/(2.0*pi)

write(1,'(e12.4)')(phi(ip), ip=1,npoin)

return

end

*---------------------------------------------------------------------*

*These next 4 subroutines construct the Hermitian Interpolation functions

*required by the semi-Lagrangian method.

*Obtained from Numerical Recipes.

*Written by F.X. Giraldo on 2/95

*---------------------------------------------------------------------*

subroutine splie2(df,f,coord,nx,ny,node,mxpoi,imax)

implicit real*8(a-h,o-z)

parameter (nmax=3000)

c

c     global arrays

c

dimension coord(mxpoi,2), f(mxpoi), df(mxpoi)

integer node(imax,imax)

c

c     local arrays

c

dimension x(nmax), y(nmax), y2(nmax)

do j=1,ny

do i=1,nx

y(i)=f(node(i,j))

x(i)=coord(node(i,j),1)

end do

call spline(y2,y,x,nx,idum,nmax)

do i=1,nx

df(node(i,j))=y2(i)

end do

end do

return

end

*---------------------------------------------------------------------*

subroutine splin2(fd,coord,f,df,xd,yd,nx,ny,node,mxpoi,imax)

implicit real*8(a-h,o-z)

parameter (nmax=3000)

c

c     global arrays

c

dimension coord(mxpoi,2), f(mxpoi), df(mxpoi)

integer node(imax,imax)

c

c     local arrays

c

dimension x(nmax), y(nmax), y2(nmax), y22(nmax)

do j=1,ny

do i=1,nx

y(i)=f(node(i,j))

y2(i)=df(node(i,j))

x(i)=coord(node(i,j),1)

end do

call splint(fd,x,y,y2,xd,nx,nmax)

y22(j)=fd

end do

c     THIS IS WHERE I DISAGREE

do j=1,ny

x(j)=coord(node(1,j),2)

y(j)=y22(j)

end do

call spline(y2,y,x,ny,idum,nmax)

call splint(fd,x,y,y2,yd,nx,nmax)

return

end

*---------------------------------------------------------------------*

subroutine spline(y2,y,x,n,idum,mx)

implicit real*8(a-h,o-z)

parameter (nmax=3000)

dimension y(mx), y2(mx), x(mx), u(nmax)

if (mx.gt.nmax) then

write(*,'(" Must expand NMAX in Subroutine Spline")')

stop

endif

y2(1)=0.0

u(1)=0.0

do i=2,n-1

sig=(x(i)-x(i-1))/(x(i+1)-x(i-1))

p=sig*y2(i-1) + 2.0

y2(i)=(sig - 1.0)/p

u(i)=(6.0*((y(i+1)-y(i))/(x(i+1)-x(i)) - (y(i)-y(i-1))

$        /(x(i)-x(i-1)))/(x(i+1)-x(i-1))-sig*u(i-1))/p

end do

y2(n)=0.0

do i=n-1,1,-1

y2(i)=y2(i)*y2(i+1) + u(i)

end do

return

end

*---------------------------------------------------------------------*

subroutine splint(yd,x,y,y2,xd,n,mx)

implicit real*8(a-h,o-z)

dimension x(mx), y(mx), y2(mx)

i1=1

i2=n

10   if (i2-i1.gt.1) then

im=(i2+i1)/2

if (xd.gt.x(im)) then

i1=im

else

i2=im

endif

goto 10

endif

x1=x(i1)

x2=x(i2)

dx=x2-x1

a=(x2-xd)/dx

b=(xd-x1)/dx

yd=a*y(i1) + b*y(i2)

$   + ((a**3-a)*y2(i1) + (b**3-b)*y2(i2))*(dx**2)/6.0

return

end

*---------------------------------------------------------------------*

*This subroutine updates the arrays PHIM,UM,VM,ALFM, PHI0,U0,V0,ALF0

*Written by F.X. Giraldo on 2.95

*---------------------------------------------------------------------*

subroutine update(phim,phi0,phip,um,u0,up,vm,v0,vp,alfm,alf0,

$                  npoin,mxpoi)

implicit real*8(a-h,o-z)

dimension phim(mxpoi), phi0(mxpoi), phip(mxpoi)

dimension um(mxpoi),   u0(mxpoi),   up(mxpoi)

dimension vm(mxpoi),   v0(mxpoi),   vp(mxpoi)

dimension alfm(mxpoi,2), alf0(mxpoi,2)

c

c     Loop through all the nodes and update

c

do ip=1,npoin

c

c     Update F(x-2*alpha,t-dt)=F(x-alpha,t)

c

phim(ip)=phi0(ip)

um(ip)=u0(ip)

vm(ip)=v0(ip)

alfm(ip,1)=alf0(ip,1)

alfm(ip,2)=alf0(ip,2)

c

c     Update F(x-alpha,t)=F(x,t+dt)

c

phi0(ip)=phip(ip)

u0(ip)=up(ip)

v0(ip)=vp(ip)

end do

return

end